A

Real-Time Research Project On

## IOT BASED OCCUPANCY MONITORING TECHNIQUES FOR ENERGY EFFICIENCT SMART BUILDINGS

(Submitted in partial fulfillment of the requirements for the award of Degree)

### BACHELOR OF TECHNOLOGY

In

### COMPUTER SCIENCE AND ENGINEERING

By

K.Santhoshi	(247R1A0590)

P.Kranthesh	(247R1A05B1)

M.Anas	(247R1A05A5)

Under the Guidance of

#### M.Madhusudhan

(Associate Professor)

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

CMR TECHNICAL CAMPUS

UGC AUTONOMOUS

(Accredited by NAAC, NBA, Permanently Affiliated to JNTUH, Approved by AICTE, New Delhi)

Recognized Under Section 2(f) & 12(B) of the UGCAct.1956, Kandlakoya (V), Medchal Road, Hyderabad-501401.

April, 2026.

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

## CERTIFICATE

This is to certify that the project entitled “IOT BASED OCCUPANCY MONITORING FOR ENERGY EFFICIENCT SMART BUILDINGS” being

submitted by K.Santhoshi (247R1A0590), P.Kranthesh (247R1A05B1) & M.Anas (247R1AO5A5) in partial fulfillment of the requirements for the award of the degree of B.Tech in Computer Science and Engineering to the Jawaharlal Nehru Technological University Hyderabad, during the year 2025-26.

The results embodied in this thesis have not been submitted to any other University or Institute for the award of any degree or diploma.

M.Madhusudhan	Dr. Nuthanakanti Bhaskar Associate Professor		HoD

INTERNAL GUIDE

## ACKNOWLEDGEMENT

We take this opportunity to express our gratitude to the people who have been instrumental in the successful completion of this project, we take this opportunity to express our profound gratitude and deep regard to our guide M.Madhusudhan, Associate Professor for his exemplary guidance, monitoring and constant encouragement throughout the project work. The blessing, help and guidance given by him shall carry us a long way in the journey of life on which we are about to embark.

We take this opportunity to extend our heartfelt appreciation to the Project Review Committee (PRC) Coordinators—Dr. J. Narasimharao, Ms. G. Swathi, Mr. G. Pavan Kumar and Mrs. A. Divya Bharathi —for their unwavering support, insightful guidance, and valuable inputs, which played a crucial role in steering this project through its various stages.

Our sincere appreciation also goes to Dr. Nuthanakanti Bhaskar, Head, for his encouragement and continuous support in ensuring the successful completion of our project.

We are deeply grateful to Dr. A. Raji Reddy, Director, for his cooperation throughout the course of this project. Additionally, we extend our profound gratitude to Sri. Ch. Gopal Reddy, Chairman, Smt. C. Vasantha Latha, Secretary and Sri. C. Abhinav Reddy, Vice-Chairman, for fostering an excellent infrastructure and a conducive learning environment that greatly contributed to our progress.

We also acknowledge and appreciate the guidance and assistance provided by the faculty and staff of CMR Technical Campus, whose contributions have been invaluable in bringing this project to fruition.

Lastly, we sincerely thank our families for their unwavering support and encouragement. We also extend our gratitude to the teaching and non-teaching staff of CMR Technical Campus for their guidance and assistance. Their contributions, along with the support of everyone who helped directly or indirectly, have been invaluable in the successful completion of this project.

K.Santhoshi (247R1A0590) P.Kranthesh (247R1A05B1) M.Anas	(247R1A05A5)

## VISION AND MISSION

### INSTITUTE VISION:

To Impart quality education in serene atmosphere thus strive for excellence in Technology and Research.

### INSTITUTE MISSION:

- To create state of art facilities for effective Teaching- Learning Process.

- Pursue and Disseminate Knowledge based research to meet the needs of Industry & Society.

- Infuse	Professional,	Ethical	and	Societal	values	among	Learning Community.

### DEPARTMENT VISION:

To provide quality education and a conducive learning environment in computer engineering that foster critical thinking, creativity, and practical problem-solving skills.

### DEPARTMENT MISSION:

- To educate the students in fundamental principles of computing and induce the skills needed to solve practical problems.

- To provide State-of-the-art computing laboratory facilities to promote industry institute interaction to enhance student’s practical knowledge.

- To inculcate self-learning abilities, team spirit, and professional ethics among the students to serve society.

## ABSTRACT

Occupancy monitoring in smart buildings addresses escalating energy demands by leveraging Internet of Things (IoT) technologies to detect and track human presence accurately. Sensors such as Passive Infrared (PIR), ultrasonic, or camera-based systems collect data, which IoT gateways transmit to cloud platforms for analysis and control of building systems like lighting and air conditioning. This approach reduces energy waste by up to 40% through automated adjustments, ensuring occupant comfort while minimizing environmental impact, with block diagrams illustrating sensor networks, data flow, and actuation mechanisms.

## LIST OF FIGURES

### FIGURE NO	FIGURE NAME	PAGE NO

Figure 3.1

Project Architecture of IoT based

Occupancy monitoring techniques	11

For energy efficienct smart buildings

Figure 3.2

Dataflow Diagram of IoT based

Occupancy monitoring techniques	14

For energy efficienct smart buildings

Figure 5.1

Hardware setup of IoT based

Occupancy monitoring techniques	26

For energy efficienct smart buildings

Figure 5.2

No Motion Detection image of of IoT based Occupancy monitoring techniques	27

For energy efficienct smart buildings

Figure 5.3

Figure 5.4

Motion Detected image of IoT based

Occupancy monitoring techniques	  28

For energy efficienct smart buildings

IoT Dashboard Output showing occupancy

status and device logs of IoT based

Occupancy monitoring techniques	                     29

For energy efficienct smart buildings

|  | LIST OF TABLES |  |
| --- | --- | --- |
| TABLE NO | TABLE NAME | PAGE NO |
| Table 6.2.1 | Power Supply Test | 31 |
| Table 6.2.2 | Sensor Testing | 31 |
| Table 6.2.3 | Display Testing | 31 |
| Table 6.2.4 | Relay And Load Control Test | 32 |
| Table 6.2.5 | IOT Communication Test | 32 |
| Table 6.2.6 | Complete System Test | 32 |

iii

TABLE OF CONTENTS

| ABSTRACT | i |
| --- | --- |
| LIST OF FIGURES | ii |
| LIST OF TABLES | iii |
| 1. INTRODUCTION | 1 |
| 1.1 PROJECT PURPOSE | 1 |
| 1.2 PROJECT FEATURES | 2 |
| 2. LITERATURE SURVEY | 3 |
| 2.1 REVIEW OF RELATED WORK | 5 |
| 2.2 DEFINITION OF PROBLEM STATEMENT | 7 |
| 2.3 EXISTING SYSTEM | 7 |
| 2.4 PROPOSED SYSTEM | 8 |
| 2.5 OBJECTIVES | 9 |
| 2.6 HARDWARE & SOFTWARE REQUIREMENTS | 10 |
| 2.6.1 HARDWARE REQUIREMENTS | 10 |
| 2.6.2 SOFTWARE REQUIREMENTS | 10 |

- SYSTEM ARCHITECTURE & DESIGN	11

  - PROJECT ARCHITECTURE	11

  - DESCRIPTION	12

  - DATA FLOW DIAGRAM	13

- IMPLEMENTATION	15

  - ALGORITHMS USED	15

  - SAMPLE CODE	19

- RESULTS & DISCUSSION	26

- VALIDATION	30

  - INTRODUCTION	30

  - TEST CASES	31

- CONCLUSION & FUTURE ASPECTS	33

  - PROJECT CONCLUSION	33

  - FUTURE ASPECTS	34

- BIBLIOGRAPHY	35

8.1               REFERENCES	35

8.2               GITHUB LINK                                                                            36

# INTRODUCTION

## INTRODUCTION

The rapid growth of urbanization and the increasing demand for sustainable infrastructure have made energy efficiency a critical concern in modern buildings. Heating, ventilation, and air conditioning (HVAC) systems, lighting, and other building services account for a significant portion of energy consumption. Traditional building management systems often operate on fixed schedules or manual controls, leading to energy wastage during periods of low or no occupancy.

The integration of Internet of Things (IoT) technologies offers a promising solution for real-time occupancy monitoring and adaptive energy management. IoT-enabled sensors—such as motion detectors, CO₂ sensors, infrared sensors, and smart cameras—can detect the presence and number of occupants in different building zones. These data can then be used to intelligently control lighting, HVAC, and other energy-consuming systems, optimizing energy use without compromising occupant comfort. Implementing IoT-based occupancy monitoring not only enhances energy efficiency, but also reduces operational costs and contributes to environmental sustainability. This approach represents a significant advancement in the development of smart, responsive buildings that are both cost-effective and eco-friendly.

### PROJECT PURPOSE

The primary purpose of this project is to enhance energy efficiency in smart buildings by implementing IoT-based occupancy monitoring techniques. Traditional building energy systems rely heavily on fixed schedules and manual controls, which are time-consuming and often ineffective in preventing unnecessary energy consumption in unoccupied spaces. By integrating IoT technologies, this project provides a more efficient, scalable, and accurate method for monitoring occupancy and controlling energy usage in real time before wastage occurs.

With the increasing demand for sustainable infrastructure and rising energy consumption in residential and commercial buildings, there is an urgent need for an intelligent system that can proactively manage energy based on actual space utilization.

By identifying real-time occupancy and usage patterns, this system automatically regulates lighting, HVAC systems, and other electrical appliances, reducing energy wastage and operational costs.

Moreover, it minimizes human intervention in energy management and improves the overall efficiency and sustainability of smart building operations.

### PROJECT FEATURES

This project incorporates several key features to improve the efficiency and accuracy of occupancy detection and energy management in smart buildings:

Real-Time Occupancy Detection: The system utilizes advanced occupancy sensors such as PIR (Passive Infrared), ultrasonic, or camera-based sensors integrated with microcontrollers to detect human presence accurately. The collected data is processed in real time to determine whether a space is occupied or vacant. This ensures precise monitoring and enables immediate control of lighting, HVAC systems, and other electrical appliances, significantly reducing unnecessary energy consumption.

Intelligent Energy Automation: Unlike traditional systems that operate on fixed schedules, this project implements automated control strategies based on real-time occupancy data. The system dynamically adjusts lighting intensity, temperature settings, and power usage according to room utilization. By analyzing occupancy patterns over time, it further optimizes energy consumption, improving overall operational efficiency and reducing electricity costs.

Scalability & Remote Monitoring: The system is designed to support large-scale deployment across residential, commercial, and industrial buildings. It leverages wireless communication technologies and cloud-based platforms for centralized monitoring and control. Building managers can access real-time data, energy usage reports, and occupancy trends remotely, enabling better decision-making and efficient facility management.

By integrating these features, this IoT-based occupancy monitoring system provides a smart, scalable, and automated solution for energy-efficient buildings, ensuring reduced energy wastage, lower operational costs, and enhanced sustainability.

# LITERATURE SURVEY

## LITERATURE SURVEY

With the rapid growth of smart infrastructure and increasing global energy demands, energy management in buildings has become a critical concern. Residential and commercial buildings account for a significant percentage of total electricity consumption, mainly due to lighting, heating, ventilation, and air conditioning (HVAC) systems. Traditional building management systems operate on fixed schedules or manual controls, often leading to excessive energy wastage in unoccupied spaces. This highlights the need for intelligent and automated occupancy-based energy management solutions.

This literature survey reviews past research on occupancy detection and energy-efficient smart building systems, highlighting existing methodologies, their strengths and weaknesses, and areas for improvement. The focus is on traditional automation systems, sensor-based occupancy detection methods, IoT architectures, wireless sensor networks, machine learning-based occupancy prediction, and hybrid intelligent energy management techniques to enhance system efficiency.

Early building automation systems relied on rule-based scheduling and manual switching mechanisms. These systems controlled lighting and HVAC systems according to predefined time intervals without considering real-time occupancy. While simple and cost-effective, they resulted in significant energy wastage. Researchers found that schedule-based control systems were inefficient in dynamic environments such as offices, classrooms, and commercial buildings.

To improve efficiency, sensor-based occupancy detection methods were introduced. Passive Infrared (PIR) sensors became widely used due to their affordability and ease of installation. Patel et al. (2018) proposed a PIR-based smart lighting system that reduced energy consumption in office environments. However, PIR sensors faced limitations in detecting stationary occupants and were prone to false triggering. Ultrasonic sensors were later introduced to improve sensitivity by detecting motion through sound wave reflections. Although ultrasonic systems improved detection accuracy, they increased system cost and complexity.

The development of Wireless Sensor Networks (WSNs) marked a significant advancement in smart building technologies. WSNs enabled distributed sensing and centralized monitoring of occupancy data across multiple building zones. Lee et al. (2019) implemented a ZigBee-based occupancy monitoring system for commercial buildings, demonstrating improved scalability and remote monitoring

capabilities. However, challenges such as signal interference, latency, and limited battery life affected large-scale deployment.

The emergence of the Internet of Things (IoT) revolutionized smart building systems by enabling real- time data collection, remote monitoring, and intelligent control. IoT-based systems integrate sensors, microcontrollers, and cloud platforms to provide automated energy management. Ahmed et al. (2020) developed an IoT-enabled smart lighting and HVAC control system that significantly reduced electricity consumption. Despite these improvements, many IoT systems lacked predictive intelligence for long-term energy optimization.

To enhance performance, researchers incorporated machine learning algorithms into occupancy detection systems. Techniques such as Decision Trees, Support Vector Machines (SVM), and Random Forest were used to classify occupancy states based on motion and environmental data. Wang et al. (2021) demonstrated that Random Forest-based occupancy prediction models improved detection accuracy compared to rule-based systems. However, these models required extensive training data and higher computational power.

Recent research has explored the use of deep learning models for occupancy prediction and energy optimization. Artificial Neural Networks (ANNs) and Long Short-Term Memory (LSTM) networks have been applied to analyze occupancy trends over time. Kumar et al. (2022) used LSTM models to forecast occupancy patterns and optimize HVAC scheduling. Although deep learning improved prediction accuracy, it introduced computational complexity and higher implementation costs.

Hybrid cloud-edge architectures have also gained attention in smart building research. Edge devices process occupancy data locally for immediate control, while cloud servers handle data analytics and long-term pattern analysis. This approach reduces latency and enhances scalability but increases system design complexity.

Multisensor fusion techniques further improved occupancy detection reliability. By combining PIR sensors, ultrasonic sensors, CO₂ sensors, and environmental data, researchers achieved higher accuracy compared to single-sensor systems. This approach reduced false positives and enhanced overall system performance.

Despite significant advancements, challenges remain in achieving high accuracy, scalability, affordability, and real-time responsiveness simultaneously. Many systems either focus only on

occupancy detection without advanced energy optimization or rely heavily on computationally intensive models.

To overcome these limitations, the proposed project integrates IoT-based occupancy monitoring with intelligent automation strategies for energy-efficient smart buildings. The system combines reliable sensors, wireless communication technologies, real-time data processing, and centralized monitoring to optimize lighting and HVAC systems based on occupancy conditions. This integrated approach enhances energy efficiency, reduces operational costs, and supports sustainable building management.

The literature survey highlights the evolution of smart building technologies from traditional rule- based systems to advanced IoT-driven and data-driven frameworks. By leveraging sensor technologies, machine learning techniques, and intelligent automation, researchers have significantly improved energy management solutions. The proposed system builds upon these advancements to deliver an efficient, scalable, and cost-effective occupancy monitoring solution.

Future research directions include improving sensor accuracy, enhancing predictive algorithms, strengthening cybersecurity measures in IoT systems, and reducing implementation costs for large- scale deployment.

## REVIEW OF RELATED WORK

The development of IoT-based occupancy monitoring systems for energy-efficient smart buildings has been extensively studied in the fields of embedded systems, wireless communication, and intelligent automation. Various approaches have been proposed, ranging from traditional automation methods to advanced IoT-integrated frameworks. This review discusses previous research and existing methodologies, highlighting their strengths and limitations.

##### Traditional Building Automation Approaches

Early building management systems relied on manual switching and fixed scheduling for lighting and HVAC control. Although simple to implement, these systems were inefficient because they did not account for real-time occupancy, leading to unnecessary energy consumption.

##### Sensor-Based Occupancy Detection

Motion sensors such as PIR and ultrasonic sensors were introduced to automate lighting and appliance control. These systems improved energy efficiency compared to manual systems but suffered from detection limitations, including false positives and inability to detect stationary occupants.

##### IoT-Based Smart Building Systems

IoT-enabled systems integrated sensors, microcontrollers, and cloud platforms for real-time monitoring and remote control. These systems enhanced scalability and centralized management but often lacked advanced predictive capabilities.

##### Machine Learning-Based Energy Optimization

Researchers incorporated machine learning algorithms such as Decision Trees, SVM, Random Forest, and Neural Networks to predict occupancy patterns and optimize energy consumption. While these approaches improved accuracy, they required large datasets and significant computational resources.

##### Hybrid Cloud-Edge Architectures

Recent advancements include hybrid systems that combine edge computing for real-time response with cloud computing for data analytics. This approach improves system efficiency and reduces latency but increases architectural complexity.

##### Comparison with the Proposed Approach

While existing methods have made significant progress in smart building automation, challenges remain in achieving a balance between accuracy, scalability, affordability, and real-time performance. The proposed approach builds upon previous research by integrating IoT-based occupancy monitoring with intelligent automation techniques. Compared to traditional rule-based systems, it enhances adaptability and energy efficiency. Unlike standalone sensor systems, it supports centralized monitoring and data-driven optimization.

This review highlights the transition from basic automation systems to intelligent IoT-driven smart building frameworks. The proposed methodology aims to address the limitations of earlier systems by providing a scalable, cost-effective, and energy-efficient occupancy monitoring solution for modern smart buildings.

### DEFINITION OF PROBLEM STATEMENT

Modern buildings waste significant energy due to lighting and HVAC systems operating in unoccupied spaces. Traditional systems use fixed schedules or manual controls that do not respond to real-time occupancy, leading to higher costs and environmental impact.

Therefore, there is a need for a cost-effective and scalable IoT-based occupancy monitoring system that can automatically detect occupancy and optimize energy usage in smart buildings.

### EXISTING SYSTEM

Conventional building energy management systems rely on manual controls such as light switches, timers, or basic motion sensors without network connectivity. These systems often result in inefficiencies, including lights and HVAC systems remaining active in unoccupied spaces or shutting off prematurely, causing discomfort and energy wastage. Such limitations can contribute to approximately 20–30% energy inefficiency and increased operational costs. Additionally, standalone controllers lack real-time data sharing, scalability, and seamless integration with advanced building management systems.

#### Limitations of Existing System

Despite advancements in smart building automation, conventional occupancy-based energy management systems suffer from the following challenges:

- Inaccurate occupancy detection: Basic motion sensors such as PIR often fail to detect stationary occupants, leading to premature shutdown of lights or HVAC systems. In some cases, appliances remain active in unoccupied areas, resulting in energy wastage and reduced user comfort.

- Lack of real-time data integration: Standalone controllers operate independently without network connectivity, preventing real-time data sharing and centralized monitoring. This limits the ability to analyze occupancy patterns and optimize energy usage effectively across the entire building.

- Limited scalability and flexibility: Traditional systems are difficult to scale for large buildings due to wiring complexity and lack of wireless communication. They also do not integrate seamlessly with advanced building management systems or cloud platforms.

- Energy inefficiency and higher operational costs: Fixed schedules and basic automation can result in approximately 20–30% energy wastage. Without intelligent analytics or predictive control, these systems fail to adapt dynamically to changing occupancy conditions, reducing overall efficiency and sustainability.

### PROPOSED SYSTEM

The proposed IoT framework deploys a network of low-power sensors (PIR, CO2, and camera modules) interfaced with microcontrollers like ESP32 for wireless data transmission to a central cloud server. Machine learning algorithms process occupancy patterns to predict and automate HVAC, lighting, and ventilation, achieving dynamic zoning and up to 50% energy reduction. The architecture includes edge computing for low latency, secure MQTT protocols, and a dashboard for remote monitoring, as depicted in the project's block diagram.

#### Advantages of the Proposed System

The proposed IoT-based occupancy monitoring system offers the following advantages:

- Energy Efficiency: The system significantly reduces energy consumption by automatically controlling HVAC, lighting, and ventilation systems based on real-time occupancy detection, achieving up to 50% energy savings.

- Intelligent Automation: Machine learning algorithms analyze occupancy patterns to enable predictive control of building systems, improving operational efficiency and minimizing manual intervention.

- Low Latency Operation: The integration of edge computing ensures faster data processing and immediate system response when changes in occupancy are detected.

- Secure Communication: The use of secure MQTT protocols ensures reliable and encrypted wireless data transmission between ESP32 microcontrollers and the central cloud server.

- Remote Monitoring and Scalability: The cloud-based dashboard enables real-time remote monitoring and control, while the wireless architecture allows easy expansion for various smart building environments.

### OBJECTIVES

- Develop an IoT-Based Occupancy Detection System – Design and implement a smart monitoring system using PIR, CO₂, and camera sensors integrated with ESP32 microcontrollers to accurately detect real-time occupancy in buildings.

- Improve Energy Efficiency – Optimize the operation of HVAC, lighting, and ventilation systems based on occupancy data to significantly reduce energy consumption in smart buildings.

- Implement Intelligent Automation – Apply machine learning algorithms to analyze occupancy patterns and enable predictive control for dynamic zoning and automated building management.

- Ensure Low Latency and Secure Communication – Integrate edge computing for faster response times and utilize secure MQTT protocols for reliable and encrypted wireless data transmission.

- Enable Remote Monitoring and Scalability – Develop a cloud-based dashboard for real-time monitoring and control while ensuring the system can scale efficiently from small offices to large commercial buildings.

## HARDWARE & SOFTWARE REQUIREMENTS

### HARDWARE REQUIREMENTS:

Hardware interfaces specify the logical characteristics of each interface between the IoT-based occupancy monitoring system and the hardware components used in the prototype. The following are some hardware requirements,

- Processor : Intel Core i3 or above

- Hard disk : 20GB

###### RAM : 4GB

- Microcontroller : ESP32 Development Board

- Display : 16x2 LCD Display

- Sensors : PIR Sensor, CO₂/Gas Sensor

- Actuator : DC Fan

- Relay Module : For Load Control

- Power Supply : 12V DC Adapter with 5V Regulator

### SOFTWARE REQUIREMENTS:

Software Requirements specify the logical characteristics of each interface and software components of the system. The following are some software requirements,

- Operating system : Windows 10 / Linux

- Language : Embedded C / Python

- Development Tool : Arduino IDE

- Back-End : Cloud Database (Firebase / MySQL)

- Protocol : MQTT

- Dashboard : Web-Based Monitoring Interface

# SYSTEM ARCHITECTURE &

DESIGN

## SYSTEM ARCHITECTURE & DESIGN

Project architecture refers to the structural framework and design of a project, encompassing its components, interactions, and overall organization. It provides a clear blueprint for development, ensuring efficiency, scalability, and alignment with project goals. Effective architecture guides the project's lifecycle, from data acquisition to intelligent control, enhancing system performance and reducing energy consumption.

### PROJECT ARCHITECTURE

This project architecture illustrates the procedure followed in the IoT-Based Occupancy Monitoring System for Energy-Efficient Smart Buildings, starting from real-time occupancy detection to automated control of building systems.

Figure 3.1: Project Architecture of IoT based occupancy monitoring        techniques for energy efficient smart buildings

### 3.2	DESCRIPTION

##### Input Data:

The system collects real-time environmental and occupancy data using PIR sensors, CO₂ sensors, and camera modules installed inside the building.

##### Data Acquisition:

Sensors continuously monitor motion, air quality (CO₂ levels), and room occupancy. The collected data is transmitted to the ESP32 microcontroller for processing.

##### Edge Processing:

The ESP32 performs preliminary data filtering and processing using edge computing techniques to reduce latency and unnecessary cloud communication.

##### Wireless Data Transmission:

Processed data is transmitted securely to the cloud server using Wi-Fi and MQTT protocol for further analysis and storage.

##### Machine Learning Analysis:

Occupancy patterns are analyzed using machine learning algorithms to predict usage trends and determine optimal control strategies for HVAC, lighting, and ventilation systems.

##### Dynamic Zoning & Automation:

Based on occupancy prediction, the system automatically controls relays connected to loads such as fans, lights, or HVAC systems, ensuring energy is used only when required.

##### Cloud Storage & Dashboard:

All occupancy data and energy usage statistics are stored in a cloud database. A web-based dashboard provides real-time monitoring, historical analysis, and remote control access.

##### Feedback Mechanism:

Continuous monitoring and stored historical data help refine occupancy prediction models, improving system accuracy and efficiency over time.

## DATA FLOW DIAGRAM

A Data Flow Diagram (DFD) is a graphical representation that illustrates how data flows within the IoT-based occupancy monitoring system, showcasing its processes, data stores, and external entities. It helps in understanding how occupancy data is collected, processed, transmitted, and used for automated control.

A Data Flow Diagram comprises four primary elements:

##### External Entities:

Building occupants, Facility Manager, Cloud Server.

##### Processes:

Occupancy Detection, Data Processing, Machine Learning Analysis, Automated Control.

##### Data Flows:

Sensor Data → ESP32 → Cloud Server → ML Processing → Control Signals → HVAC/Lighting Systems.

##### Data Stores:

Cloud Database, Local Memory Storage.

These components are represented using standardized symbols, such as circles for processes, arrows for data flows, rectangles for external entities, and open-ended rectangles for data stores.

#### Benefits:

The visual nature of DFDs makes them understandable to both technical and non-technical stakeholders. They help in identifying system boundaries, optimizing energy usage, improving automation efficiency, and ensuring secure data transmission.

#### Applications:

DFDs are widely used in smart building management, IoT system development, energy optimization systems, and automation control systems. They help in mapping occupancy workflows and reducing unnecessary energy consumption.

#### Levels of DFD:

DFDs are structured hierarchically:

##### Level 0 (Context Diagram):

Provides a high-level overview of the entire occupancy monitoring system, showing interactions between occupants, IoT devices, cloud server, and building systems.

##### Level 1:

Breaks down processes into sub-processes such as data acquisition, processing, cloud communication, and automated control.

##### Level 2+:

Provides detailed representation of machine learning analysis, dynamic zoning logic, and HVAC control mechanisms.

Figure 3.2: Dataflow Diagram of IoT based occupancy monitoring techniques for energy efficienct smart buildings

# IMPLEMENTATION

## IMPLEMENTATION

The implementation phase of the project involves the practical deployment of the IoT-based occupancy monitoring system in a smart building environment. It includes hardware integration, sensor data acquisition, wireless communication, cloud connectivity, machine learning analysis, and automated control of building systems. Proper coordination between hardware and software components ensures efficient energy management, real-time monitoring, and intelligent automation within the designed constraints.

## ALGORITHMS USED

#### Sensor-Based Occupancy Detection Algorithm

The system uses PIR sensors, CO₂ sensors, and optional camera modules to detect occupancy in indoor environments.

- PIR sensors detect motion based on infrared radiation changes.

- CO₂ sensors measure indoor air quality and occupancy density.

- Sensor fusion techniques combine multiple sensor readings to improve detection accuracy.

#### Advantages:

- Simple and cost-effective implementation.

- Real-time detection capability.

- Low power consumption.

#### Disadvantages:

- PIR sensors may fail to detect stationary occupants.

- CO₂ levels may vary due to environmental factors.

- Camera-based detection may raise privacy concerns.

#### Machine Learning for Occupancy Prediction

To improve system intelligence, machine learning algorithms are used to analyze historical occupancy data and predict future usage patterns.

The system applies supervised learning models such as:

- Random Forest (RF)

- Support Vector Machine (SVM)

- Decision Tree Classifier

These models analyze time-series occupancy data to predict peak usage hours and automate HVAC and lighting accordingly.

#### Advantages:

- Improves energy efficiency through predictive automation.

- Learns occupancy patterns over time.

- Enhances dynamic zoning of smart buildings.

#### Disadvantages:

- Requires sufficient historical data for training.

- Performance depends on data quality.

- May require retraining for seasonal changes.

#### Edge Computing Algorithm

Edge processing is implemented on the ESP32 microcontroller to reduce latency and minimize cloud dependency.

- Local filtering of sensor data.

- Threshold-based decision making for quick response.

- Reduces unnecessary data transmission to cloud server.

#### Advantages:

- Faster response time.

- Reduced bandwidth usage.

- Improved system reliability.

#### Disadvantages:

- Limited computational capability of microcontroller.

- Complex algorithms cannot be fully executed at edge level.

#### MQTT-Based Secure Communication Protocol

The system uses MQTT (Message Queuing Telemetry Transport) protocol for lightweight and secure data communication between ESP32 and cloud server.

#### Why MQTT?

- Low bandwidth consumption.

- Reliable publish-subscribe architecture.

- Suitable for IoT applications.

#### How MQTT Fits in the System?

- Sensor data is published from ESP32.

- Cloud server subscribes and stores data.

- Control commands are sent back to actuators.

#### Automated Control Algorithm for HVAC & Lighting

Based on occupancy prediction results, control algorithms activate relays connected to:

- Lighting systems

- Ventilation systems

- Cooling fans (HVAC prototype) The algorithm ensures:

- Devices turn ON when occupancy is detected.

- Devices turn OFF during vacancy.

- Dynamic zoning to reduce unnecessary energy consumption.

To implement this project, we have designed the following modules:

##### Hardware Setup Module:

This module integrates ESP8266/NodeMCU, PIR sensor, relay module, LCD display, and fan (load) to form the complete IoT-based occupancy monitoring system.

##### Occupancy Detection Module:

The PIR sensor detects human movement and sends signals to the microcontroller to identify whether the room is occupied or empty.

##### Processing & Control Module:

The microcontroller processes sensor data and automatically controls the relay to switch appliances ON or OFF based on occupancy.

##### Display Module:

The LCD displays real-time status such as “Room Empty” or “Occupancy Detected”.

##### IoT Monitoring Module:

Occupancy and appliance status data can be monitored remotely through an IoT platform for analysis.

##### Energy Efficiency Module:

This module ensures energy saving by automatically turning OFF appliances when no occupancy is detected.

### SAMPLE CODE

#include <LiquidCrystal.h> #include <stdio.h>

//LiquidCrystal lcd(6, 7, 5, 4, 3, 2);

LiquidCrystal lcd(13, 12, 14, 27, 26, 25);

#include <WiFi.h> #include <HTTPClient.h>

HTTPClient http;

const char *ssid = "iotserver";

const char *password = "iotserver123";

int httpResponseCode;

String servername = "http://projectsfactoryserver.in/storedata.php?name="; String accountname = "iot1687";

String field1 = "&s1="; String field2 = "&s2="; String field3 = "&s3="; String payload="";

unsigned char rcv,count,gchr,gchr1,robos='s';

int sti=0;

String inputString = "";	// a string to hold incoming data boolean stringComplete = false; // whether the string is complete

int buzzer = 23;

int ir	= 18;

int light = 21; int fan  = 19;

#define RXD2 16

#define TXD2 17

void beep()

{

digitalWrite(buzzer, LOW);delay(2000);digitalWrite(buzzer, HIGH);

}

void okcheck()

{

unsigned char rcr; do{

rcr = Serial.read();

}while(rcr != 'K');

}

int sts1=0,sts2=0,sts3=0,sts4=0,cntlmk=0; int stn=0;

String iot_string="";

void iot_send()

{

lcd.setCursor(15,1);lcd.print("U");

http.begin(servername + accountname + field1 + iot_string); httpResponseCode = http.GET();

if(httpResponseCode>0)

{

payload="";

//Serial.print("HTTP Response code: ");

//Serial.println(httpResponseCode); payload = http.getString();

//Serial.println(payload);

}

else

{

;

//Serial.print("Error code: ");

//Serial.println(httpResponseCode);

}

delay(8000); lcd.setCursor(15,1);lcd.print(" ");

}

void setup()

{

Serial.begin(9600);//serialEvent(); Serial2.begin(9600, SERIAL_8N1, RXD2, TXD2);

pinMode(ir, INPUT); pinMode(buzzer, OUTPUT);

pinMode(light, OUTPUT);pinMode(fan, OUTPUT);

digitalWrite(buzzer, HIGH);

digitalWrite(light, LOW);digitalWrite(fan, LOW);

lcd.begin(16, 2); lcd.print("IOT Occupancy"); lcd.setCursor(0,1); lcd.print(" Monitoring"); delay(2500);

WiFi.begin(ssid, password);

Serial.println("Connecting"); while(WiFi.status() != WL_CONNECTED)

{

delay(500);

}

//Serial.println(WiFi.localIP()); delay(3000);

lcd.clear();

}

void loop()

{

iot_string=""; if(digitalRead(ir) == LOW)

{

lcd.setCursor(0,0);lcd.print("Occupancy "); iot_string="Occupancy_Light_Fan_ON"; digitalWrite(light, HIGH);digitalWrite(fan, HIGH);

iot_send(); while(digitalRead(ir) == LOW);

}

if(digitalRead(ir) == HIGH)

{

lcd.setCursor(0,0);lcd.print("No Occupancy "); iot_string="No_Occupancy_Light_Fan_OFF"; digitalWrite(light, LOW);digitalWrite(fan, LOW);

iot_send(); while(digitalRead(ir) == HIGH);

}

delay(1000);

}

void serialEvent()

{

while (Serial.available())

{

char inChar = (char)Serial.read(); if(inChar == '*')

{

gchr = Serial.read();

}

if(inChar == '#')

{

gchr1 = Serial.read();

}

}

}

void converts(unsigned int value)

{

unsigned int a,b,c,d,e,f,g,h;

a=value/10000; b=value%10000; c=b/1000; d=b%1000;

e=d/100; f=d%100;

g=f/10; h=f%10;

a=a|0x30; c=c|0x30; e=e|0x30; g=g|0x30; h=h|0x30;

Serial.write(a); Serial.write(c); Serial.write(e); Serial.write(g); Serial.write(h);

}

void convertl(unsigned int value)

{

unsigned int a,b,c,d,e,f,g,h;

a=value/10000; b=value%10000; c=b/1000; d=b%1000;

e=d/100; f=d%100;

g=f/10; h=f%10;

a=a|0x30; c=c|0x30; e=e|0x30; g=g|0x30; h=h|0x30;

// lcd.write(a); lcd.write(c);

lcd.write(e);

lcd.write(g);

lcd.write(h);

}

# RESULTS & DISCUSSION

## RESULTS & DISCUSSION

The following screenshots present the hardware implementation and real-time working model of the proposed IoT-Based Occupancy Monitoring Techniques for Energy-Efficient Smart Buildings system. These images demonstrate the successful integration of sensors, microcontroller, relay modules, and output devices used for occupancy detection and automatic energy control. The results validate that the system efficiently detects occupancy and controls electrical appliances to reduce unnecessary power consumption.

#### Hardware Setup of the Proposed System

The below figure shows the complete hardware setup of the IoT-based occupancy monitoring system.

Figure 5.1 : Hardware Setup IoT based occupancy monitoring techniques for energy efficienct smart buildings

#### Occupancy Detection – No Motion Condition:

In this stage, the PIR sensor does not detect any human movement in the room. The microcontroller processes the sensor data and identifies the room as unoccupied.

System behavior in this condition:

- Occupancy Count = 0

- Relay remains OFF

- Fan/Load remains OFF

- LCD displays “No Occupancy” or “Room Empty”

This condition ensures that electrical appliances are automatically turned OFF when the room is empty, thereby preventing energy wastage.

Figure 5.2 : No Motion Detection image of IoT based occupancy monitoring techniques for energy efficienct smart buildings

#### Occupancy Detection – Motion Detected Condition:

In this stage, when a person enters the detection area, the PIR sensor detects motion. The microcontroller processes the input and activates the relay module. System behavior in this condition:

- Occupancy detected

- Relay turns ON

- Fan/Load turns ON

- LCD displays “Occupancy Detected”

This confirms that the system successfully automates energy control based on real-time occupancy detection.

Figure 5.3 : Motion Detected image of IoT based occupancy monitoring techniques for energy efficienct smart buildings.

CMRTC                                                                                                                                                         28

#### IoT Dashboard Output:

- In this stage, the system sends occupancy data to the IoT server through the NodeMCU module. The data is displayed on a web page in tabular format with occupancy status and time.Occupancy detected

- System behavior in this condition:
• Occupancy status is updated on the webpage
• When motion is detected → “Occupancy_Light_Fan_ON” is displayed
• When no motion → “No_Occupancy_Light_Fan_OFF” is displayed
• Date and time of each event is recorded

- This shows that the system successfully monitors and stores real-time occupancy data on the IoT dashboard.

Figure 5.4 : IoT Dashboard Output showing occupancy status and device logs of IoT based

occupancy monitoring techniques for energy efficienct smart buildings.

CMRTC                                                                                                                                                29

# VALIDATION

## VALIDATION

The validation of this IoT-Based Occupancy Monitoring System is carried out through systematic testing and performance evaluation to ensure accurate occupancy detection and efficient energy management. The process includes sensor testing, dataset validation, machine learning performance analysis, and real-time deployment assessment. A structured validation approach ensures that the system minimizes false occupancy detection, reduces energy wastage, and maintains reliable automation under different environmental conditions.

###### INTRODUCTION

The rapid growth of urbanization and the increasing demand for sustainable infrastructure have made energy efficiency a critical concern in modern buildings. Heating, ventilation, and air conditioning (HVAC) systems, lighting, and other building services account for a significant portion of energy consumption. Traditional building management systems often operate on fixed schedules or manual controls, leading to energy wastage during periods of low or no occupancy.

The integration of Internet of Things (IoT) technologies offers a promising solution for real-time occupancy monitoring and adaptive energy management. IoT-enabled sensors—such as motion detectors, CO₂ sensors, infrared sensors, and smart cameras—can detect the presence and number of occupants in different building zones. These data can then be used to intelligently control lighting, HVAC, and other energy-consuming systems, optimizing energy use without compromising occupant comfort.

Implementing IoT-based occupancy monitoring not only enhances energy efficiency, but also reduces operational costs and contributes to environmental sustainability. This approach represents a significant advancement in the development of smart, responsive buildings that are both cost-effective and eco- friendly.

## 6.2	TEST CASES

TABLE 6.2.1 – POWER SUPPLY TEST

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | Power ON Test | To verify system power supply | Adapter connected to circuit and switched ON | Power LED glows, system starts |
| 2 | Voltage Stability Test | To check proper voltage supply | Continuous power supply for 10 minutes | No system restart or failure |

TABLE 6.2.2 – SENSOR TESTING

| Test case ID | Test case name | Purpose | Input | Output |
| --- | --- | --- | --- | --- |
| 1 | Motion Detection Test | To check PIR sensor functionality | Person moves in front of sensor | Motion detected signal sent |
| 2 | No Motion Test | To verify no false detection | No movement in room | No detection signal |

TABLE 6.2.3 – DISPLAY TESTING

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | LCD Initialization Test | To verify LCD working | System powered ON | Welcome message displayed |
| 2 | Occupancy Status Display | To display occupancy result | Motion detected | "Room Occupied" shown on LCD |
| 3 | Empty Room Display | To show empty status | No motion detected | "Room Empty" displayed |

TABLE 6.2.4 – RELAY AND LOAD CONTROL TEST

| Test case ID | Test case name | Purpose | Input | Output |
| --- | --- | --- | --- | --- |
| 1 | Relay ON Test | To check relay switching | Occupancy detected | Relay activated |

| Test case ID | Test case name | Purpose | Input | Output |
| --- | --- | --- | --- | --- |
| 2 | Fan ON Test | To verify load control | Relay activated | Fan turns ON |
| 3 | Relay OFF Test | To check automatic OFF | No motion for 5 minutes | Relay deactivated |
| 4 | Fan OFF Test | To verify energy saving | Relay OFF | Fan turns OFF |

TABLE 6.2.5 – IOT COMMUNICATION TEST

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | WiFi Connection Test | To verify internet connection | NodeMCU connects to WiFi | Connected successfully |
| 2 | Data Upload Test | To check cloud update | Occupancy detected | Data updated on IoT dashboard |
| 3 | Network Failure Test | To verify error handling | WiFi disconnected | Error message / Local storage enabled |

TABLE 6.2.6 – COMPLETE SYSTEM TEST

| Test case ID | Test case name | Purpose | Test Case | Output |
| --- | --- | --- | --- | --- |
| 1 | Full System Test | To verify integrated working | Person enters and exits room | Sensor detects → LCD updates → Relay activates → Fan ON/OFF automatically |

# CONCLUSION & FUTURE ASPECTS

## CONCLUSION & FUTURE ASPECTS

In conclusion, the IoT-Based Occupancy Monitoring Techniques for Energy-Efficient Smart Buildings project successfully demonstrates an intelligent system that monitors occupancy in real time and controls electrical appliances to reduce energy wastage and improve efficiency. By integrating sensors, microcontrollers, and IoT technology, the system ensures automated and optimized energy management within buildings. In the future, the project can be enhanced by incorporating Artificial Intelligence for predictive analysis, cloud-based monitoring for remote access, and large-scale deployment in commercial infrastructures, further improving sustainability and smart energy management.

### PROJECT CONCLUSION

In conclusion, IoT-based occupancy monitoring techniques play a pivotal role in enhancing the energy efficiency of smart buildings by providing real-time data on space utilization and occupancy patterns. By integrating sensors, IoT devices, and intelligent analytics, these systems enable automated control of lighting, HVAC, and other energy-consuming systems, reducing wastage and operational costs.

Additionally, such techniques improve occupant comfort and safety while offering scalable and flexible solutions adaptable to diverse building types. The adoption of IoT-driven occupancy monitoring not only contributes to sustainable building management but also supports data-driven decision-making for facility operators. Future advancements, including AI integration and predictive analytics, are expected to further optimize energy usage and create smarter, more responsive building environments.

### 7.2	FUTURE ASPECTS

The proposed IoT-based occupancy monitoring system for energy-efficient smart buildings has demonstrated effective real-time occupancy detection and optimized energy management. However, there is significant scope for further enhancement to improve scalability, intelligence, and real-world applicability. Future advancements in IoT, Artificial Intelligence, cloud computing, and sustainable technologies can make the system more efficient, adaptive, and reliable.

These are some future aspects: Integration of Artificial Intelligence for predictive occupancy analysis, Real-Time Cloud-Based Monitoring and Data Analytics, Large-Scale Deployment in Commercial and Smart City Infrastructure, Integration with Renewable Energy Sources, Development of a Mobile Application for Remote Control and Monitoring, Enhancement of Sensor Accuracy and Multi-Sensor Fusion Techniques, Implementation of Adaptive Energy Optimization Algorithms, and Integration with Security and Building Automation Systems.

# BIBLIOGRAPHY

## BIBLIOGRAPHY

### REFERENCES

- Kemal Akkaya, Ismail Guvenc, Ramazan Aygun, Nezih Pala & Abdullah Kadri. IoT-based Occupancy Monitoring Techniques for Energy-Efficient Smart Buildings. IEEE Wireless Communications and Networking Conference Workshops (WCNCW), 2015.

- Mohammad Jabirullah Almas Khan, Mohd Zohaib Ali, Syed Sirajuddin Wajih & Mohd Azhar Hussain. IoT-Based Occupancy Monitoring Techniques for Energy Efficient Smart Buildings. Turkish Online Journal of Qualitative Inquiry (TOJQI), Vol. 12, No. 3, 2021.

- P. F. Karjou, et al. Practical Design and Implementation of IoT-Based Occupancy Monitoring Systems for Office Buildings: A Case Study. Energy and Buildings, 2024.

- Pratiksha Chaudhari, Yang Xiao, Mark Ming-Cheng Cheng & Tieshan Li. Fundamentals, Algorithms, and Technologies of Occupancy Detection for Smart Buildings Using IoT Sensors. Sensors, 2024.

- Azad Shokrollahi, Jan A. Persson, Reza Malekian, Arezoo Sarkheyli-Hägele & Fredrik Karlsson. Passive Infrared Sensor-Based Occupancy Monitoring in Smart Buildings: A Review of Methodologies and Machine Learning Approaches. Sensors, 2023.

- “IoT-Based Systems for Energy Efficiency in Smart Buildings: Present, Trends and Sustainability Perspective,” Mehmet Yagiz Orhan & Aytac Ugur Yerden. International Journal of Advanced Natural Sciences and Engineering Researches.

- A. B. Barral, Tomás Domínguez-Bolaño, Carlos J. Escudero & José A. García-Naya. An IoT System for Smart Building Combining Multiple mmWave FMCW Radars Applied to People Counting. arXiv, 2024.

- Zhirui Luo, Ruobin Qi, Qingqing Li, Jun Zheng & Sihua Shao. ABODE-Net: An Attention-Based Deep Learning Model for Non-Intrusive Building Occupancy Detection Using Smart Meter Data. arXiv, 2022.

- Sangkeum Lee, Sarvar Hussain Nengroo, Hojun Jin, Yoonmee Doh, Chungho Lee, Taewook Heo & Dongsoo Har. Power Management in Smart Residential Building with Deep Learning Model for Occupancy Detection by Usage Pattern of Electric Appliances. arXiv, 2022.

  - GITHUB LINK

https://github.com/santhoshikammari/iot-occupancy-monitoring.git